N-X10 User Manual

Factory default

| Programming password | 123456 (default) |
| --- | --- |
| Open door mode | Card or digital password |
| Lock relay time | Can set (00~99)S |
| Tamper alarm | Open |
| Open card function | Yes |

Indicator light:

- Open the door, the green light is on

- Enter the programming mode, the green light is always on

- Standby state, no light

- The card is not registered, when punch by card, the light turn red.

- The door open password is wrong, the red light is on.

Buzzer:

Valid command: 3 short “beep beep beep”

Invalid command: long “beep”

Full record: 3 long “beep...beep...beep....

Press [*] to cancel the command: 3 short “beep beep beep” quickly

Features and settings:

- How to change administrator code? (Default: 123456)

Ans: - Press * # administrator code (Default: 123456) >> Press 0 >> Press 6 digit administrator code >> Again press 6 digit administrator code >> The device will beep 3 times, the setting is successful >> ESC (**).

- How to add card in device?

Ans: - Press * # administrator code (Default: 123456) >> Press 5 >> Press 3 digit user ID >> Show card in device >> ESC (**).

Register more cards and repeat the above steps (Example: Press * # administrator code (Default: 123456) press 5 >>      Press 3 digit user ID 001 swipe card >> 002 swipe card >> 003 swipe card... for continuous registration) >> ESC (**).

- How to delete card in device?

Ans: - Press * # administrator code (Default: 123456) >> Press 7 >> Show card in device >> ESC (**).

Delete more cards and repeat the above steps (Example: Press * # Administrator code (Default: 123456) press 6 >> 000, 001, 002... for continuous deletion) >> ESC (**).

- How to set password? (Only 2 password support in device)

Ans: - Press * # administrator code (Default: 123456) >> Press 3 >> Press 4 digit password >> Then again press 3 >> Press 4 Digit same password >> The device will beep 3 times, the setting is successful >> ESC (**).

- How to delete all password?

Ans: - Press * # administrator code (Default: 123456) >> Press 3 >> 0000 >> The device will beep 3 times, the setting is successful >> ESC (**).

- How to set lock delay time?

Ans: - Press * # administrator code (Default: 123456) >> Press 2 >> Set lock delay second (01 to 99) >> ESC (**).

- Restore factory defaults?

Ans: - Press * # administrator code (Default: 123456) >> Press 197320 >> ESC (**) The device will beep 3 times, then 3 beeps, wait for about 20 seconds, all setting parameters and data will be restored to the factory default.

(Note: - Please use with caution, all registered data will be deleted and cleared after recovery all user settings).